| STENCH! STENCH! Duke OESO Guidelines for Safe Use of β-mercaptoethanol (2-ME) Complete Lab-Specific Safety Information on page 2. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Hazards | Potential Hazards | - Fatal in contact with skin, category 2 and therefore Particularly Hazardous. Rapidly absorbed through skin. Toxic if inhaled or ingested. - Low odor threshold (0.12-0.64 ppm). Very unpleasant stench similar to odorant in natural gas. Misuse or spill may cause suspicion of natural gas leak. - May cause allergic skin reaction. Causes eye damage and skin irritation. - May cause liver or heart damage with repeated oral exposure. |  |  |  |
| Hazard Controls | Selection & Purchase | - Purchase the smallest, shatter resistant containers, at the lowest concentration feasible. - Consider alternate methods and use a less hazardous substance if possible. - Purchase butyl or laminate gloves for clean-up of small spills. |  |  |  |
|  | Storage & Transport | - Store away from metals, oxidizers, and any other incompatibles. - Storage container must be tightly closed, resealed and stored upright. - Store in sealed secondary container in a well ventilated area (not in a cold room). Do not store on the floor. |  |  |  |
|  | Engineering Controls | - Always work with 2-ME in a chemical fume hood or 100% exhausted biological safety cabinet (Class II, Type B2). |  | - Eyewash required in immediate work area. Eyewash-drench hose preferred. |  |
|  | Work Practice Controls | - Work with the smallest practicable amount for the experiment being performed. - Designate a specific work area for 2-ME (within a hood or exhausted BSC) and label it. - Keep containers closed as much as possible when not in use. - Plan work to avoid glove contact. - Clean work area & reusable equipment with soap, water and paper towels; dispose of paper towels as chemical waste. If stench remains, neutralize with an aqueous solution of sodium hypochlorite (bleach). |  |  |  |
|  | Personal Protective Equipment (PPE) | - Wear closed-toed shoes and clothing covering the legs. x2 Minimum PPE: - Buttoned lab coat - Safety goggles - TWO pairs nitrile gloves. - Change gloves immediately if splashed. |  |  | If glove contact is expected or |
|  |  | - Risk of splash: ADD: chemical resistant sleeves & apron, face shield, and laminate or butyl gloves. |  |  |  |
| Other | Emergencies | - See Emergency Response webpage or flip chart and/or lab specific chemical hygiene plan. - For clean-up of small (<10 ml) spills confined to the chemical fume hood, wear butyl or laminate gloves & other PPE noted above. Clean up with inert absorbent materials (i.e., vermiculite, dry sand). Clean spill area as indicated in Work Practice Controls section. Contaminated PPE and clean-up materials must be double-bagged and tightly sealed. Dispose as chemical waste. - Other spills: Contact OESO Spill Response Team via Duke Police. |  |  |  |
|  | Waste | Store accumulating waste in hood until picked up by OESO. See also lab-specific chemical hygiene plan, Lab Chemical Waste Management Practice, and Drain Disposal Practice. |  |  |  |
|  | Training | Sign signature page in lab-specific chemical hygiene plan to indicate review. |  |  |  |
|  | Questions | Contact OESO Lab Safety at 919-684-8822 or labsafety@dm.duke.edu. |  |  |  |

| STENCH! |  | Lab-Specific Safety Information for β-mercaptoethanol (2-ME) Supplements the Guidelines for Safe Use of 2-ME |  |  | STENCH! |
| --- | --- | --- | --- | --- | --- |
| Lab | PI Name | Click or tap here to enter PI Name |  |  |  |
|  | Location | Enter building(s) and room(s) where lab is located |  |  |  |
| Lab-Specific Hazard Controls | Purchase Details | Maximum container size | Enter maximum container sized purchased |  |  |
|  |  | Maximum concentration | Enter maximum concentration purchased |  |  |
|  |  | Container type | Enter the container material |  |  |
|  |  | Specific product information | Enter supplier name/product number or purity/grade to purchase |  |  |
|  | Storage | Specific location | Enter specific storage location | Not in a cold room! |  |
|  | Use Information | Designated work area (specific room(s) and area(s)) | Enter rooms and areas designated for use |  |  |
|  |  | Maximum quantity | Enter maximum quantity to be used at a time |  |  |
|  |  | Location of supplies for decontamination | Bleach: Enter location of bleach Paper Towels: Enter location of paper towels |  |  |
|  |  | Location of supplies for small spill clean-up (Use decon supplies if needed) | ☐Laminate gloves: Location of Laminate Gloves (i.e. SilverShield) ☐Butyl gloves: Location of butyl gloves Location & Type of inert absorbent: Location of vermiculite or dry sand |  |  |
|  | Details of Process | - Enter steps used in lab process(es) or experiment(s) |  |  |  |