| Duke OESO Guidelines for Safe Use of Formalin, Formaldehyde Solutions, and Paraformaldehyde Complete Lab-Specific Safety Information on page 2. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Facts | - Formaldehyde (CAS 50-00-0) is H2CO. This is the simplest aldehyde and is a gas at room temperature. - Formalin is a saturated formaldehyde solution (usually 37%), so 10% formalin is roughly 3.74% formaldehyde. - Some solutions have methanol added to stop polymerization; these solutions may be flammable or combustible. - Paraformaldehyde; PFA (CAS 30525-89-4) is polymerized formaldehyde. PFA is used to make very pure formaldehyde solutions or can be heated to create pure formaldehyde gas. |  |  |  |  |  |  |  |
| Hazards | Potential Hazards - Formaldehyde | - Acutely toxic via ingestion, inhalation, and skin contact. - Causes skin corrosion and serious eye damage. - Skin sensitizer; may cause an allergic skin reaction. |  |  | - Suspected mutagen. - Carcinogen. |  |  |  |
| Hazard Controls | Selection & Purchase | - Use a safer alternative when possible. - Purchase the smallest containers at the lowest concentration practical. - If storing a large amount (>2L), purchase spill kit with formaldehyde neutralizer or solidifier. |  |  |  |  |  |  |
|  | Storage & Transportation | - Store with flammables in tightly-closed shatter-resistant containers. - Store away from oxidizers, reducing agents, metals, and acids (not in cold room). - Use secondary container for transport. |  |  |  |  |  |  |
|  | Engineering Controls | - Eyewash required in immediate work area. - Eyewash-drench hose preferred. - Safety shower may be required for large quantities. |  | - Work with concentrated solutions (>4%) and all solids only in a chemical fume hood. - Dilute solutions (<4%) can be used on benchtop in small quantities if containers are opened only briefly. |  |  |  |  |
|  | Work Practice Controls | Formaldehyde Work Area: Danger! Toxic! Formaldehyde Work Area: Danger! Toxic! Labs handling >100 ml regularly on the benchtop must contact OESO Lab Safety for exposure assessment. - Tare (pre-weigh) empty container with lid. - Go to hood, add powder, close lid. - Go to balance to weigh. - Return to hood. - Tare (pre-weigh) empty container with lid. - Go to hood, add powder, close lid. - Go to balance to weigh. - Return to hood. Designate a work area for formaldehyde and label it. ↗ - Keep containers closed as much as possible. - Line work area with absorbent, leak-proof bench pads. - If weighing, place balance in hood OR use Tare Method → - Decontaminate with soap and water solution. |  |  |  |  |  |  |
|  | Personal Protective Equipment | Minimum PPE: - Nitrile (or latex) gloves (Change immediately if contaminated and every 2 hours. Wash hands at time of change.) - Splash goggles - Fully buttoned lab coat with sleeves extending to the wrists. |  |  |  |  |  |  |
|  |  | Risk of splash/large amounts: (in addition to the above, wear) - Face shield. - Tyvek sleeves and/or gown/apron. |  |  |  |  |  |  |
| Other | Emergencies | See Emergency Response webpage or flip chart and/or lab specific chemical hygiene plan. Contact OESO Spill team for spills outside fume hood if there is eye or respiratory irritation. |  |  |  |  |  |  |
|  | Waste | See lab-specific chemical hygiene plan. ≤10% formalin (≤3.75% formaldehyde) can be disposed down the drain. PFA & solutions must be collected as chemical waste. |  |  |  |  |  |  |
|  | Training | Sign signature page in Lab-Specific Chemical Hygiene Plan to indicate review and follow lab-specific instructions for “Online Training” below. |  |  |  |  |  |  |
|  | Questions | Contact OESO Lab Safety at 919-684-8822. |  |  |  |  |  |  |

| Lab-Specific Safety Information for Formalin, Formaldehyde Solutions, and Paraformaldehyde Supplements the Guidelines for Safe Use of Formalin, Formaldehyde solutions, and PFA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Lab | PI Name | Click or tap here to enter PI Name |  |  |  |
|  | Location | Enter building(s) and room(s) where lab is located |  |  |  |
| Lab-Specific Hazard Controls | Purchase Details | Select Type of Product | 10% Formalin or 10% Neutral Buffered Formalin (NBF) ☐ | 37% Formaldehyde solution ☐ | Paraformaldehyde Solid and/or PFA solution ☐ |
|  |  | Maximum container size | Enter maximum container size purchased | Enter maximum container size purchased | Enter maximum container size purchased |
|  |  | Container type | Enter the container material | Enter the container material | Enter the container material |
|  |  | Specific product information | Enter supplier name/product number or purity/grade to purchase | Enter supplier name/product number or purity/grade to purchase | Enter supplier name/product number or purity/grade to purchase |
|  | Storage | Specific location Not in cold room! | Enter specific storage location | Enter specific storage location | Enter specific storage location |
|  | Use Information | Designated work area | Enter rooms and areas designated for use | Enter rooms and areas designated for use | Enter rooms and areas designated for use |
|  |  | Max. quantity to use at a time | Enter maximum quantity | Enter maximum quantity | Enter maximum quantity |
|  |  | Max. conc. to use at a time | Enter maximum % if different than 10% | Enter maximum % if different than 37% | Enter maximum % if solution |
|  |  | Location and type of spill supplies | Spill supply type and location |  |  |
|  | Waste Disposal | Select Method of Disposal for Chemical Waste | ☐ Drain Disposal ☐ Waste Collection: Collection Location | ☐ Drain Disposal ONLY if final USE conc. is <3.8% ☐ Waste Collection: Collection Location | NO Drain Disposal ☐ Waste Collection: Collection Location |
|  | Online Training | Formaldehyde Exposure Awareness | ☒ Recommended by OESO for all users before starting work ☐ Required by PI for all users before starting work ☐ Required by OSHA for users in Clinical, Pathology, and Anatomy labs exposed to >0.1 ppm formaldehyde in air (see training for specific scenarios) |  |  |
|  |  | Formaldehyde Hazard Communication | ☐ Required by PI for all users before starting work ☐ Required by OSHA for users in Clinical, Pathology, and Anatomy labs exposed to <0.1 ppm formaldehyde in air (see training for specific scenarios) |  |  |
|  | Details of Process | Click or tap here to enter details specific to your lab’s process with formalin, formaldehyde solutions, and/or paraformaldehyde. |  |  |  |