| Duke OESO Guidelines for Safe Use of Osmium Tetroxide (OsO4) Lab-specific Safety Information on page 2 MUST BE APPROVED in advance by OESO and the PI. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hazards | Potential Hazards | - Solid is fatal if inhaled (category 1) or if swallowed or in contact with skin (category 2). - 2% and 4% solutions are fatal if inhaled (category 1), category 3 oral and category 4 dermal. Solutions are also repiratory sensitizers (category 1), serious eye damage (category 1) and skin irritation (category 2). High risk. - Causes severe skin burns and eye damage; lachrymator. Has an unpleasant chlorine-like odor and also has poor warning properties (odor detectable only above exposure limit). - May cause allergy or asthma symptoms or breathing difficulties if inhaled. - Symptoms may be delayed. - Exposure limit (OSHA): 0.002mg/m3 (0.0002 ppm) – 8 hr TWA; Short term (NIOSH, ACGIH):0.0006 ppm.Chronic exposure causes damage to the liver and kidney. - Sublimes readily at room temp and significantly when refrigerated. - Consult the Safety Data Sheet (SDS) and the Laboratory Chemical Safety Summary. |  |  |  |  |  |  |
| Hazard Controls Hazard Controls | Selection & Purchase | - Use a less dangerous product if possible. - Purchase in solution or in pre-weighed ampule. | - Purchase corn oil for decontamination and spill clean-up. |  |  |  |  |  |
|  | Storage & Transportation | - Solid andolutions: store and transport in sealed, glass (OsO4 penetrates plastic), shatter-resistant containers within secondary containment. Use Teflon tape to reseal any solutions. - Store in a dry and well-ventilated place; keep storage area secure. - Container must be kept away from strong reducing agents, organic materials, powdered metals, and hydrochloric acid. |  |  |  |  |  |  |
|  | Engineering Controls | - Eyewash (preferably with drench hose) is required. - A safety shower is highly recommended. |  | - All work must be done in a chemical fume hood. NO BSCs! - Open only inside hood. |  |  |  |  |
|  | Work Practice Controls | - Designate a specific area for osmium tetroxide use and label it. - Keep containers closed and tightly sealed as much as possible. - If possible, purchase as solution OR make the entire ampule into a solution and then aliquot the solution for use. - If weighing is necessary, place balance in hood OR use Tare Method |  |  |  | Osmium Tetroxide Work Area Danger! Toxic! Corrosive! - Tare (pre-weigh) empty glass container with lid. - Go to hood, add solid, close lid. - Go to balance to weigh. - Return to hood. |  |  |
|  |  | - Decontaminate containers, equipment, & surfaces with corn oil. If OsO4 is still present, the oil will turn black. Use this as an indicator to ensure decon is complete. |  |  |  |  |  |  |
|  | Personal Protective Equipment | Minimum PPE: - Safety goggles and face shield (8” length min.) - If using the solid, use 2 pairs nitrile gloves (inspect for defects before use) - If using solutions, 1 pair of nitrile gloves will be sufficient. - Avoid glove contact & change gloves at least every 30 minutes. - Fully buttoned lab coat with sleeves extending to the wrists. |  |  |  |  | x2 |  |
|  |  | Risk of splash (or for spill clean-up): - Chemical-resistant sleeves and apron over lab coat |  |  |  |  |  |  |
|  |  | Check the manufacturer’s glove guide for glove effectiveness if using a solvent other than water. |  |  |  |  |  |  |

| Other | Medical Emergencies | - See Emergency Response webpage or flip chart and/or lab specific chemical hygiene plan - Exposed persons: seek immediate medical attention at the nearest Emergency Room. |
| --- | --- | --- |
|  | Spills | - Spill >1 g or outside hood: Call 911 from a campus phone or 919-684-2444 from any phone. - Spills < 1 g in hood: Cover spill with corn oil-soaked inert absorbent. Scoop, double bag & seal. - AFTER the spill is absorbed, wipe area with corn oil, then soap and water. |
|  | Waste | OsO4 is P-listed waste. Accumulate separately. Submit empty containers (including syringes) as waste if OsO4 was the only active ingredient in the container. NO DRAIN DISPOSAL. See lab-specific chemical hygiene plan and Lab Chemical Waste Management Practice for more information. |
|  | Training | Sign lab-specific Chemical Hygiene Plan to indicate review. |
|  | Questions | Contact OESO Lab Safety at 919-684-8822 or labsafety@dm.duke.edu. |

|  |  | Lab-Specific Safety Information for Osmium Tetroxide Supplements the Guidelines for Safe Use of Osmium Tetroxide Must be approved by OESO and PI below. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lab Info | PI Name | Click or tap here to enter PI Name |  | PI Approval (signature): Date: Click or tap to enter a date. |  |  |
|  | Location | Enter building(s) and room(s) where lab is located |  |  |  |  |
|  | OESO approval | Click or tap here to enter OESO Reviewer |  | Signature: Date: Click or tap to enter a date. |  |  |
| Lab-Specific Hazard Controls | Purchase Details | Maximum container size | Enter maximum container size purchased |  |  |  |
|  |  | Maximum concentration | Enter maximum concentration purchased |  |  |  |
|  |  | Container type | Enter the container material |  |  |  |
|  |  | Specific product info. | Enter supplier name/product number or purity/grade to purchase |  |  |  |
|  | Storage | Specific location | Enter rooms and areas designated for storage |  |  |  |
|  | Use Information | Designated work area (specific room(s) and area(s)) | Enter rooms and areas designated for use |  |  | Label work area! |
|  |  | Type of container to use | Type of container in which OsO4 is used or stored in the lab |  |  | NO PLASTIC! |
|  |  | Maximum quantity | Enter maximum quantity to be used at a time |  |  |  |
|  |  | Location of supplies for decontamination (or spill clean-up) | Corn Oil: List Location Inert Absorbent: List Location |  |  |  |
|  | Waste Information | Details about waste - location, type of container | Enter waste collection details – location, type of glass container Empty containers are submitted as waste |  |  | Accumulate Separately! No plastic! |
|  | Details of Process | - Enter steps used in lab process(es) or experiment(s) |  |  |  |  |