Reviewed by OESO:

Standard Operating Procedure

for work with

| Chemical name/class: |  |  | Pyrophoric and other highly reactive materials (Customized SOP required – see instructions in red) | CAS #: |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PI: |  |  |  | Date: |  |  |
| Building: |  |  |  | Room #: |  |  |
|  |  | Designated Work Area: |  |  |  |  |

This SOP has been approved by the following Principal Investigator:

| Name: |  | Signature: |  | Date: |  |
| --- | --- | --- | --- | --- | --- |

- Circumstances of Use:

This SOP must be customized for each pyrophoric material in use in the lab. Use this section to describe the circumstances of use.

- Potential Hazards:

| Pyrophorics are chemicals that, even in small quantities, are liable to ignite within five minutes after coming into contact with air. Check the Safety Data Sheet (SDS) to see if the material presents other hazards, such as corrosivity, teratogenicity, water reactivity, peroxide formation, or systemic effects. If other hazards are present, appropriate safety precautions should be addressed in this SOP. Some classes of pyrophoric materials are listed below: |  |
| --- | --- |
| Some Classes of Pyrophoric Chemicals | Examples of class |
| Grignard reagents | RMgX |
| Metal alkyls and aryls | RLi (tert-butyl lithium is very pyrophoric), RNa, R3Al, R2Zn |
| Metal carbonyls | Ni(CO)4, Fe(CO)5, Co2(CO)8 |
| Alkali metals | Na, K |
| Metal powders | Al, Ca, Co, Fe, Mg, Mn, Pd, Pt, Ti, Sn, Zn, Zr |
| Metal hydrides | NaH, LiAlH4 |
| Nonmetal hydrides | B2H6 and other boranes, PH3, AsH3, SiH4 |
| Nonmetal alkyls | R3B, R3P, R3As |
| Phosphorus (white) |  |

- Engineering Controls:

- Liquid pyrophorics should be stored in sealed containers with PTFE-lined septa to prevent air exposure, and manipulated via syringe or cannula in a chemical fume hood (over a spill tray if possible) with the sash as low as possible. - Solid pyrophorics must be handled only in an inert atmosphere glove box or glove bag. - Mineral oil bubblers must be used to release pressure from reagent or reaction vessels. - Use a blast shield if available.

- Work Practice Controls:

Lab-specific written procedures, approved in advance by the Principal Investigator (signature required at top of customized SOP) and reviewed by OESO (as a High Risk Procedure), are required. These must include a designated work area (at top of template) and cover all sections of this template. Before working with these compounds, read the SDS and other reference material carefully. Good web resources include: - UCSD training videos - Lab Safety Workplace – Handling Pyrophoric Materials training course - Sigma Aldrich Technical Bulletins AL-164 (Handling Pyrophoric Reagents) and AL-134 (Handling Air-Sensitive Reagents). Other Aldrich Technical Bulletins describe specific laboratory equipment designed for use with air- and moisture-sensitive reagents.) - Prudent Practices in the Laboratory (National Academies Press) Sections 4.D and 6.G and Laboratory Chemical Safety Summaries for - Butyllithiums - Sodium (reactive metal) - Lithium aluminum hydride - Before purchasing pyrophoric materials, consult with OESO Fire Safety at 919-684-5609 to select a compatible fire extinguisher. The extinguisher must be on hand before any work with the pyrophoric material takes place (and preferably before the material is ordered). - Purchase minimal amounts of pyrophoric materials. - Set up a designated area for work with pyrophoric materials – a chemical fume hood and/or a (dry) glove box (with inert atmosphere, if needed) located within 10 seconds of an eyewash/drench hose, safety shower, and an appropriate fire extinguisher, as determined in consultation with Fire Safety. - Incompatible materials should be removed from the area. - A container of powdered lime or sand should be kept within arm’s reach (for covering spills). - Employees should be aware of the location of all emergency equipment and should know how to use it if needed. - Store and use pyrophoric chemicals under an inert atmosphere or under kerosene as appropriate. - Know the location of the nearest compatible fire extinguisher and how to use it. - See the above-referenced Aldrich technical bulletins for recommendations on safe transfer of liquid pyrophorics. - Before conducting the actual procedure, always perform a dry run (without the pyrophoric material) to identify and resolve possible safety hazards. - Work within sight and/or hearing of at least one other person who is familiar with the hazards and written procedures.

- Personal protective equipment (PPE):

Wear a fully buttoned, flame-resistant lab coat (Nomex material or equivalent) with sleeves extended to the wrists, along with safety goggles and standard nitrile laboratory gloves. Leather or Kevlar gloves are recommended beneath nitrile gloves (for fire protection). If large quantities will be used, a chemical-resistant apron is also required. Note that personal clothing should not be of a type that may ignite (such as polyester or nylon).

- Transportation and Storage:

- Store and use pyrophoric chemicals under an inert atmosphere or under kerosene as appropriate. - Store in secondary containers, away from flammables and oxidizers. (You may be able to reuse the secondary container provided by the manufacturer.) - Avoid areas with heat, flames, and water. - Some of these materials may need to be kept below threshold temperatures.

- Waste Disposal:

Waste pyrophorics should be disposed of immediately through OESO Environmental Programs (they should NOT be allowed to accumulate), following your laboratory-specific chemical hygiene plan and the requirements of Duke University’s Chemical Waste Policy and the Laboratory Chemical Waste Management Practices.

- Exposures/Unintended contact:

Contact Employee Occupational Health and Wellness (EOHW) at 919-684-3136 for medical advice on occupational chemical exposures. For an actual chemical exposure or injury, complete the work-related injury or illness report found at: http://www.hr.duke.edu/benefits/medical/workcomp/report.php. If medical attention is needed, see http://www.hr.duke.edu/benefits/medical/workcomp/medical.php.

- Emergency/Spill Procedures:

In the case of a release of pyrophoric material, fire, or explosion in the lab, leave the area immediately and contact Duke Police by calling 911 from a campus phone or 919-684-2444 from any phone. Duke Police will contact the fire department, facilities and/or OESO as needed.

- Training of personnel:

Users must be given hands-on training for procedures/experiments involving pyrophoric or other highly reactive materials before beginning work. Hands-on fire extinguisher training is highly recommended. Contact OESO Fire Safety at 919-684-5609 for more information or to set this up. All personnel are required to complete the online General Lab Safety session thru the OESO website. This session includes an introduction to general chemical safety. Furthermore, training on these specific procedures must be performed by the PI or knowledgeable designee for all personnel working with pyrophoric materials, and must be documented (topics covered, date, employee names and signatures). All personnel shall read and fully adhere to the laboratory- and chemical-specific SOP for any explosives, and shall document that they have read it by signing and dating the SOP.

“I have read and understand this SOP. I agree to fully adhere to its requirements.”

| Last | First | Duke ID | Signature | Date |
| --- | --- | --- | --- | --- |